# 13.3 EQUIPMENT STORAGE

Equipment is stored in a way that prevents contamination of the fruit.

Equipment must be stored to prevent fruit contamination.

• Ensure all picking equipment is clean, maintained, and sanitised (where required) before use.
• Use only food-grade cleaners and sanitisers, and store them to avoid contaminating fruit.
• Keep any tools that contact chemicals separate from equipment that contacts fruit.
• Do not store anything other than fruit in bins, or use picking gloves/bags for non-produce purposes.

# CALIBRATION

• Always calibrate new jugs—do not assume accuracy (a GGAP sticker is not sufficient).
• Replace scale batteries if readings become inaccurate and recalibrate.
• Contractors applying PPPs or foliar fertiliser are responsible for calibrating their own equipment.

# Scales And Jugs

Scales
• Zero the scales.
• Verify accuracy using either: A calibrated test weight, or
Comparison with another calibrated scale using any weight.
• If inaccurate, repair or replace the scales.
• Record calibration on the sheet or on the scale itself and sign off.

Jugs
• Place the jug on calibrated scales and zero.
• Fill to the 1-L mark with water (1 L = 1 kg).
• If reading ≠ 1 kg: Replace the jug or Adjust water level until scales read 1 kg and mark the correct line.
• Record calibration on the sheet or on the jug.

# Fertiliser Spreaders

Granular Fertiliser Calibration

- Add a measured amount (A), e.g., 5 kg.

- Select an aperture setting and drive at a constant speed until empty (distance = B).

- Measure width of spread (C).

- Calculate:
– Area covered = C × B
– 10,000 ÷ (C × B) = multiplier
– Multiplier × A = kg/ha applied

Foliar Fertilisers & Fertigation
• Foliar: calibrated using the spray unit procedure.
• Fertigation: calibrate per manufacturer instructions.

# Irrigation System

Follow irrigation consultant/manufacturer guidelines and document outputs.

Procedure:

- Prepare: Place containers randomly in the row/zone. Select a run time (e.g., 15 minutes).

- Run: Operate system for the chosen time. Measure and record water volume in each container.

- Calculate: Average volume per container. Convert to application rate per hour (adjust if using minutes).

- Adjust & Repeat: Increase or decrease run time as needed.  Re-test until the desired application rate is achieved.

Considerations:

- Uniformity—large differences may indicate blockages or malfunctioning sprinklers.

- Calibrate each irrigation zone separately.

- Tree age affects required irrigation area.

- Ensure system pressure is within recommended range.

- Maintain filters, valves, sprinkler heads, and other components regularly.

Please read about Irrigation Calibration Quick Test and worksheet by clicking the link below.

Microsoft Word - IRRIG8QuickWorksheet-DripMicro.doc

## Spray Equipment

- Spray equipment is kept in good working condition and calibrated annually.

- This verification shall be certified or documented either by participation in an official scheme (where it exists) or by having been carried out by a person who can demonstrate their competency.

- The calibration requirement applies to both air blast sprayers and knapsack units.

#### Knapsack Sprayers and or backpack sprayers worksheet can be found under 13.4 in the appendix.

#### Air-Blast Sprayers

The Calibration formulae sheet from the GrowSafe website can be found by clicking the link below.

https://www.growsafe.co.nz/Growsafe/Growsafe2017/Rsrc/FormsAndTemplates/ApplicationAndCalibration.aspx

The orchard sprayer calibration worksheet from the Growsafe website can be found by clicking the link below. https://www.growsafe.co.nz/Growsafe/Growsafe2017/Rsrc/FormsAndTemplates/ApplicationAndCalibration.aspx