# 32.11 PLANT PROTECTION PRODUCTS STORAGE

## PPROPRIATE STORAGE OF PPPs

- Store PPPs according to regional council rules and label requirements.

- Keep storage cool, secure, weather-proof, and on impervious shelving.

- Segregate PPPs from foliar fertilisers; powders above liquids.

- Group similar products (herbicides, fungicides, insecticides) where practical.

- Keep a spill kit on site (no sawdust).

- Provide accurate measuring/mixing equipment.

- Store PPPs in original containers or clearly labelled replacements.

- Store only approved products; segregate obsolete ones for disposal.

- Display emergency procedures, phone numbers, and hazard signage within 10 m.

- Keep a fire extinguisher accessible.

- Maintain a monthly stock inventory, showing incoming/outgoing products and maximum quantities.

- Inventory must include product name, location, UN number, quantity, hazard info, storage requirements, SDS availability.

- Store the inventory away from the shed and ensure emergency services can access it.

- A first aid kit must be available.

Useful tools:
https://www.hazardoussubstances.govt.nz/calculator 
https://www.growsafe.co.nz/StandardManual/Mana/Inventory.aspx

## Structural Requirements

- Storage must be sound, ventilated, secure, fire-resistant, weather-proof, and well lit.

- Constructed to keep rodents and birds out.

- Must contain contents for 30 minutes in case of fire.

## Avoiding Risk & Cross-Contamination

- Keep the shed locked when not in use.

- Provide a decontamination area (hose/running water).

- Store powders above liquids and use impervious shelving to prevent contamination.

## Temperature

- Store PPPs at temperatures recommended on the product label.

## Illumination

- Lighting must allow all labels to be read clearly.

## Spill Containment

- Storage must have a bunded, impervious floor capable of containing 125% of stored liquid volume, or use bunded containers.

## Agrichemical Storage Checklist

The requirements listed below apply to the pesticide storage area on this site. The grey areas do not apply for fertiliser storage.

| Agrichemical Storage Checklist | Fertiliser Store | PPP Store |
| --- | --- | --- |
| Plant Protection products (PPP) are stored in accordance with regional council regulations |  |  |
| Bunded impervious floor that fully contains where possible 125% of liquids stored in the event of spillage or leakage |  |  |
| Ventilated, sound, secure, fire-resistant and weather-proof facilities with adequate light |  |  |
| Controlled access and locked at all times when not in use |  |  |
| Storage is appropriate to temperature conditions |  |  |
| Impervious shelving; |  |  |
| Pesticides are segregated from foliar fertilizers |  |  |
| Powders are stored above liquids |  |  |
| Herbicides, insecticides, fungicides or bactericides grouped where practicable |  |  |
| Have decontamination facility at the Crop Protection Product storage shed (hose and running water) |  |  |
| Have a spill kit (equipment and approved absorbent material available for cleaning up spills) with the location signposted |  |  |
| Have facilities and equipment for accurately measuring and mixing sprays; |  |  |
| Store pesticides/chemicals always in their original containers; where storing in original containers is impractical due to container condition/size or storage space condition, store in suitable containers, label clearly with a copy of the original container label and treat accordingly |  |  |
| Only currently approved chemicals are stored on site, or obsolete chemicals segregated and prepared for disposal |  |  |
| Have signage listing accident procedures and emergency telephone numbers displayed within 10 meters from the shed |  |  |
| Have hazard signs displayed on shed wall |  |  |
| Have the location of the nearest accessible telephone listed on signage |  |  |
| Have ready access to a fire extinguisher |  |  |
| Have an inventory record of stock on hand updated at least every month; - Inventory must show incoming and outgoing products. - Inventory is updated within a month from incoming/outgoing product. Daily updates (after transfers) are recommended to give better control. - Inventory is expressed in units, e.g. boxes/containers as a minimum; kgs and litres is more accurate. - Balance the stock in the chemical shed with the inventory list regularly (minimum is annually; monthly is better practice. |  |  |
| Have access to a first aid kit |  |  |
| The spray shed is constructed to prevent access of rodents, birds and other animals |  |  |