13.5 KNAPSACK SPRAYERS AND/OR BACKPACK SPRAYERS WORKSHEET

| What is the desired water application rate? (From the product label) | L ______ / ______ |
| --- | --- |
| Select the appropriate chemical rate (from the label). | mL ______ / ______ |
| Record the nozzle type and size (from the equipment). | Type Size |
| Is there a required operating pressure? If so, what is it? | Yes No kPa or Bar ______ |
| Tank size of equipment. | ______ L |
| Hand-held height above target should be recorded. | ______ cm above target |

Step 2

| Measure out an area 10 m × 1 m (this is 10 m2). This is an area of 1/1000th of a hectare (1 ha = 100 m × 100 m). |  |
| --- | --- |
| Using the required pressure (or a constant pressure), time how long it takes to spray this area. (Walk at a comfortable pace, ensuring complete coverage.) | Time in seconds |
| Spray into a measuring jug for the same time it took to cover the 10 m × 1 m area. Record this output. | ________ L |
| You can now multiply this figure by 10, which will give a water application rate per 100 m2. | 10 × ________ L =______ / 100 m2 |
| OR you can now multiply this figure by 1000 to give a water application rate/ha. | 1000 × ______ /L = ______ /ha |

Step 3

Does this water application rate comply with the chemical label requirements? ⃞ Yes ⃞ No

If not, suggest a way to change this rate to meet the requirements on the label. .............................................................................................................................................. ..............................................................................................................................................